COMP6011 Research Task 1 - Research Notes

AI Perception Module for Freight Truck ADAS

Curtin University | Semester 1, 2026

----------------------------------------------------------------------

Week 1 Notes - Understanding the Problem

Date: 4 March 2026

Focus: Initial problem reading and scoping

What I understood from the assignment brief

The task is to design a perception module for a freight truck that uses a single forward-facing camera. The camera sees the road in front and the system needs to detect things like other vehicles, pedestrians, cyclists, lane lines, and the drivable area in real time.

This is different from normal car-based systems because the truck camera sits much higher (around 2.5 to 3.5 metres) compared to a regular dashcam which is around 1.2 metres. This changes how objects look in the image.

Why this matters - Australian context

I looked up Australian truck crash statistics. Found that 188 people died in heavy truck crashes in the 12 months to March 2023. This is the real-world problem we are trying to help solve. The key challenges specific to Australia are:

- Long outback highways with no lane markings

- Kangaroos and livestock on roads - not in any dataset

- Road trains (triple-trailer trucks) which are unique to Australia

- Extreme sun glare on flat roads

- Red dust reducing camera visibility

Key technical question I noted

The brief says monocular camera only. This means no LiDAR, no stereo cameras. I need to find out how accurate detection and depth estimation can be from a single camera. Initial reading suggests this is a known hard problem - called monocular 3D detection.

Initial reading list

- Elhassan et al. 2024 - recommended by teacher - covers CNN and Transformer segmentation

- YOLOP paper - first multi-task model on embedded hardware

- BDD100K paper - the most relevant dataset

- BITRE crash statistics reports

----------------------------------------------------------------------

Week 2 Notes - Literature Review

Date: 11 March 2026

Focus: Reading papers and building taxonomy

Paper 1: Elhassan et al. 2024 - Main Survey Paper

Title: Real-time semantic segmentation for autonomous driving: A review of CNNs, Transformers, and Beyond

Journal: Journal of King Saud University - Computer and Information Sciences

This is the core paper we discussed with the teacher. Main things I learned:

- There are 4 main architecture families: encoder-decoder CNNs, two-branch CNNs, multi-branch CNNs, and Transformers

- All benchmarks are on Cityscapes and CamVid using RTX 3090 - this is a problem for embedded deployment

- Best real-time CNN: PIDNet-M at 80.1% mIoU, 40 FPS

- Best speed-accuracy balance: LCFNet-slim at 78.95% mIoU, 114 FPS

- Per-class gap is the biggest finding - cyclists only get 60-68% IoU vs 93%+ for road

Research Gaps I identified from Elhassan et al. 2024

These are the gaps I will discuss in my report:

- Gap 1: All benchmarks on desktop GPU only - no embedded testing

- Gap 2: Only European/US datasets - no Australian roads

- Gap 3: Small object gap is critical but not solved

- Gap 4: No multi-task coverage in this paper

- Gap 5: No adverse weather testing

Paper 2: YOLOP (Wu et al. 2022)

Title: YOLOP: You Only Look Once for Panoptic Driving Perception

Journal: Machine Intelligence Research

This is the key paper for multi-task perception. The big deal is it was the FIRST model to do all three tasks (detection, drivable area, lane detection) at the same time on embedded hardware. 23 FPS on Jetson TX2.

Architecture: one shared CSPDarknet encoder plus three separate decoder heads. Trained on BDD100K.

Detection result: 76.5% mAP@50

Drivable area: 91.5% mIoU

Lane detection: 70.5% accuracy

Paper 3: YOLOPv3 (Zhao et al. 2024)

Title: YOLOPv3: Multi-Task Visual Perception for Object Detection and Segmentation

Journal: Remote Sensing

This is the current state of the art. Key improvements over original YOLOP:

- Multi-scale high-resolution features - helps with small objects

- Self-attention module in the lane detection head - captures long-range context

- Only 12 anchors vs 45 in HybridNets - more memory efficient

- End-to-end training - no complicated stage-wise setup

Results: 84.3% mAP, 93.2% mIoU, 88.3% lane accuracy on BDD100K

Architecture families summary table (my notes)

ENet/ERFNet: Ultra fast but low accuracy - not suitable for safety

BiSeNet/DDRNet: Two-branch - good balance, most popular for ADAS

PIDNet: Three-branch - best CNN accuracy, uses PID control idea

SegFormer: Transformer - global context but too slow for edge

YOLOP family: Multi-task - best overall for our use case

----------------------------------------------------------------------

Week 3 Notes - Benchmarking

Date: 18 March 2026

Focus: Running experiment and comparing benchmark numbers

My experiment setup

I ran a preliminary experiment on Google Colab using BDD100K validation images. I tested three models: YOLOv8s, YOLOv12n, and YOLOP detection head only.

Dataset subset: 2,500 images from BDD100K validation split

Classes tested: Vehicle, Pedestrian, Cyclist, Traffic Sign

Hardware: Google Colab T4 GPU

Experiment results - my findings

YOLOv8s results:

- Vehicle: 81.3% mAP

- Pedestrian: 48.7% mAP

- Cyclist: 41.2% mAP - CRITICAL GAP

- Latency: 8.3ms

YOLOv12n results:

- Vehicle: 83.7% mAP

- Pedestrian: 51.4% mAP

- Cyclist: 44.8% mAP - still a critical gap

- Latency: 13.9ms

YOLOP detection head only:

- Vehicle: 77.4% mAP

- Pedestrian: 44.1% mAP

- Cyclist: 38.6% mAP - lowest but this is only one head

- Latency: 11.2ms

Key observation from experiment

The cyclist class is critically under-performing across ALL models. 38-45% mAP compared to 77-84% for vehicles. This 35-40 percentage point gap is the most important finding in my report. On Australian roads where cyclists use road shoulders on freight routes this is a real safety problem.

Cityscapes benchmark - from literature

I collected these numbers from Elhassan et al. 2024 for the report tables:

- ENet: 58.3% mIoU, 135 FPS - too inaccurate

- DDRNet-23-S: 77.4% mIoU, 107 FPS - good balance

- PIDNet-S: 78.6% mIoU, 93 FPS - recommended

- LCFNet-slim: 78.95% mIoU, 114 FPS - best speed+accuracy

- PIDNet-M: 80.1% mIoU, 40 FPS - highest accuracy

GPU vs CPU latency issue

Important note: when I ran the YOLOv8 inference cell the first time I got 454ms latency and only 2 FPS. This was because I was running on CPU not GPU. After switching to T4 GPU in Colab the latency dropped to around 8-12ms and 80-120 FPS. This is an important lesson - always check you are using GPU when testing deep learning models.

----------------------------------------------------------------------

Week 4 Notes - Methodology and Implementation Plan

Date: 24 March 2026

Focus: Finalising method selection and writing methodology section

Why I chose YOLOPv3

After comparing all models I decided YOLOPv3 is the best choice. My reasoning:

- Best results on BDD100K across all three tasks simultaneously

- BDD100K is the most relevant dataset - has weather, night, highway

- The YOLOP architecture family is the only one proven on Jetson embedded hardware

- End-to-end training is simpler to implement and maintain

- Only 12 anchors makes it more memory efficient than HybridNets

Five customisations I am proposing

These are the changes needed to make YOLOPv3 work for an Australian freight truck specifically:

- C1 Camera height: Truck camera is at 2.5-3.5m not 1.2m. Need to recalibrate the anchor boxes to match the truck perspective geometry. Objects appear smaller and the horizon is further away.

- C2 Class reweighting: Use focal loss with higher weights on cyclist and pedestrian classes. This forces the model to pay more attention to the hard-to-detect safety-critical classes. Gamma=2.0, alpha=0.75 for these classes.

- C3 Australian adaptation: Fine-tune on 2000 labelled Australian highway images. This is needed because all current training data is from Europe or USA. Will use SimCLR self-supervised learning first on unlabelled images, then supervised fine-tuning.

- C4 Weather augmentation: Add synthetic rain, dust haze, night conditions, and sun glare to training images. The literature shows 15-20% accuracy drop in rain without this. These conditions are common on Australian freight routes.

- C5 INT8 quantisation: Use TensorRT INT8 to make the model 2-3x faster on Jetson. This is necessary to hit the 25+ FPS threshold for real-time safety operation.

Training plan

I designed a three-phase training approach:

- Phase 1: Pre-train segmentation heads on Cityscapes for 100 epochs

- Phase 2: Joint multi-task training on BDD100K for 300 epochs

- Phase 3: Fine-tune on Australian highway dataset for 50 epochs with lower learning rate

Data collection plan for Australian dataset

This is something I am proposing as part of the implementation plan. Key routes to cover:

- Hume Highway NSW-VIC

- Bruce Highway QLD

- Great Northern Highway WA

Need to capture all conditions: day, night, dawn, dusk, rain, dust, clear. Target around 50 hours of footage and annotate 2000 images using Label Studio.

----------------------------------------------------------------------

Week 5 Notes - Ethics, Discussion, Writing Up

Date: 26 March 2026

Focus: AI ethics section and final report writing

Australia AI Ethics Principles - my notes on each

- Principle 1 Human wellbeing: System designed to reduce crashes. But risk of driver over-reliance - must keep human in the loop

- Principle 2 Human-centred values: Cyclists get lower accuracy - this is an equity problem. C2 focal loss addresses this

- Principle 3 Fairness: Datasets are all Western cities. Remote Australian areas are under-represented. Need to collect from rural areas specifically

- Principle 4 Privacy: Camera records identifiable people on public roads. Processing must be on-device. No footage sent to cloud. Privacy Act 1988 applies

- Principle 5 Reliability: Need a fallback if GPU fails. Watchdog system to switch to basic rule-based detection

- Principle 6 Transparency: Detection outputs are visual overlays - human-readable. Confidence scores visible to fleet managers

- Principle 7 Contestability: Fleet operators need a way to report false detections. Near-miss logs must feed back into retraining

- Principle 8 Accountability: Legal responsibility must be defined between model developer, fleet operator, and vehicle manufacturer

Carbon footprint calculation

Used the ML CO2 Impact Calculator to estimate training emissions. Training on one NVIDIA A100 in an Australian data centre (0.49 kg CO2 per kWh grid intensity):

- Phase 1 Cityscapes 100 epochs: approximately 3-4 kg CO2

- Phase 2 BDD100K 300 epochs: approximately 12-16 kg CO2

- Phase 3 fine-tuning 50 epochs: approximately 2-3 kg CO2

- Total estimated: 18-25 kg CO2e

Limitations I need to acknowledge

- My experiment was on Colab T4 not actual Jetson hardware

- Australian dataset has not been collected yet - this is a future step

- Domain adaptation results are estimated not measured

- The synthetic driving scene I created for the notebook gave wrong detections - cyclist was misclassified as tennis racket because the model is trained on real photos not cartoons

Final writing checklist

- Abstract - done

- Introduction with BITRE statistics - done

- Literature review with 4 subsections - done

- Benchmarking tables and figures - done

- Methodology with 5 customisations - done

- AI Ethics 8 principles - done

- Discussion including SDGs and carbon - done

- Conclusion linking to each RO - done

- References in APA7 - done

- Appendices with video link and GitHub - pending

----------------------------------------------------------------------

Final Reflections

The most important thing I learned from this assignment is that benchmark performance does not equal real-world performance. All these models perform well on Cityscapes and BDD100K but none of them have been tested on Australian roads. The domain gap is a serious problem that would not be obvious if you only looked at published papers.

The cyclist detection gap was the finding that surprised me the most. Even the best models only get 38-45% mAP for cyclists. This means in my experiment approximately 1 in every 2 cyclists would not be detected. On a freight route in regional Australia where cyclists use road shoulders this is genuinely dangerous.

The multi-task approach is clearly the right direction. YOLOPv3 doing all three tasks in one pass is more practical than running three separate models, uses less memory, and actually performs better because the tasks help each other learn.

Notebook link: [INSERT GITHUB LINK]

Video link: [INSERT ECHO360 LINK]